VLSI DESIGN INTERNSHIP – TASK 3

Verilog RTL Design of Sequential Circuits and Flip-Flops

Submitted by: Shruti Vyas

# 1. D Flip-Flop

RTL Code

module d_ff(input clk, input reset, input d, output reg q);
always @(posedge clk) begin
    if (reset) q <= 1'b0;
    else       q <= d;
end
endmodule

Testbench

module tb_d_ff;
reg clk, reset, d;
wire q;

d_ff uut(.clk(clk),.reset(reset),.d(d),.q(q));

initial begin
    clk=0;
    forever #5 clk=~clk;
end

initial begin
    reset=1; d=0;
    #10 reset=0;
    #10 d=1;
    #10 d=0;
    #10 d=1;
    #10 $finish;
end
endmodule

Figure 1: D Flip-Flop waveform

# 2. JK Flip-Flop

RTL Code

module jk_ff(input clk, input reset, input j, input k, output reg q);
always @(posedge clk) begin
    if (reset) q <= 1'b0;
    else case ({j,k})
        2'b00: q <= q;
        2'b01: q <= 1'b0;
        2'b10: q <= 1'b1;
        2'b11: q <= ~q;
    endcase
end
endmodule

Testbench

module tb_jk_ff;
reg clk, reset, j, k;
wire q;

jk_ff uut(.clk(clk),.reset(reset),.j(j),.k(k),.q(q));

initial begin
    clk=0;
    forever #5 clk=~clk;
end

initial begin
    reset=1; j=0; k=0;
    #10 reset=0;
    #10 j=1; k=0;   // set
    #10 j=0; k=1;   // reset
    #10 j=1; k=1;   // toggle
    #20 $finish;
end
endmodule

Figure 2: JK Flip-Flop waveform

# 3. 4-bit Register

RTL Code

module register_4bit(
    input clk, input reset,
    input [3:0] d, output reg [3:0] q
);
always @(posedge clk) begin
    if (reset) q <= 4'b0000;
    else       q <= d;
end
endmodule

Testbench

module tb_register_4bit;
reg clk, reset;
reg [3:0] d;
wire [3:0] q;

register_4bit uut(.clk(clk),.reset(reset),.d(d),.q(q));

initial begin
    clk=0;
    forever #5 clk=~clk;
end

initial begin
    reset=1; d=4'b0000;
    #10 reset=0;
    #10 d=4'b1010;
    #10 d=4'b0101;
    #10 d=4'b1111;
    #10 $finish;
end
endmodule

Figure 3: 4-bit Register waveform

# 4. 4-bit Up Counter

RTL Code

module counter_4bit(
    input clk, input reset,
    output reg [3:0] count
);
always @(posedge clk) begin
    if (reset) count <= 4'b0000;
    else       count <= count + 1'b1;
end
endmodule

Testbench

module tb_counter_4bit;
reg clk, reset;
wire [3:0] count;

counter_4bit uut(.clk(clk),.reset(reset),.count(count));

initial begin
    clk=0;
    forever #5 clk=~clk;
end

initial begin
    reset=1;
    #10 reset=0;
    #80 $finish;
end
endmodule

Figure 4: 4-bit Up Counter waveform

# 5. Result

The Verilog RTL modules and their testbenches were prepared for the required sequential circuits. The waveforms show the expected clock-driven behavior of the flip-flops, register, and counter.

# 6. Conclusion

The task was completed by implementing sequential circuits in Verilog HDL, creating testbenches, and verifying their operation through simulation waveforms.